[image: image1.jpg]landscape




7800 E. Michigan Ave.

Kalamazoo, Michigan  49048

Phone: (800) 521-2546

Fax: (269) 381-3455

www.landscapeforms.com
specify@landscapeforms.com
Chipman Table

Product Guide Specification

Specifier Notes:  This product guide specification is written according to the Construction Specifications Institute (CSI) 3-Part Format, including MasterFormat, SectionFormat, and PageFormat, as described in The Project Resource Manual—CSI Manual of Practice, Fifth Edition.
The section must be carefully reviewed and edited by the Architect to meet the requirements of the project and local building code.  Coordinate this section with other specification sections and the Drawings.  Delete all “Specifier Notes” after editing this section.

Section numbers are from MasterFormat 2010 Edition.
SECTION 12 93 43.53  SITE SEATING AND TABLES
Specifier Notes:  This section covers Landscape Forms, Inc. “Chipman” tables.  Consult Landscape Forms, Inc. for assistance in editing this section for the specific application.
PART 1
GENERAL

1.1
SECTION INCLUDES
A.
Tables.
1.2
REFERENCES
Specifier Notes:  List standards referenced in this section, complete with designations and titles.  Delete standards not included in the edited section.  This article does not require compliance with standards, but is merely a listing of those used.
A.
ASTM Testing Standards: 
1.
ASTM B 117 – Standard Practice for Operating Salt Spray (Fog) Apparatus.

2.
ASTM D 522 – Standard Test Methods for Mandrel Bend Test of Attached Organic Coatings.

3.
ASTM D 523 – Standard Test Method for Specular Gloss.

4.
ASTM D 2247 – Standard Practice for Testing Water Resistance of Coatings in 100% Relative Humidity.

5.
ASTM D 2794 – Standard Test Method for Resistance of Organic Coatings to the Effects of Rapid Deformation (Impact).
6.
ASTM D 3359 – Standard Test Methods for Measuring Adhesion by Tape Test.
7.
ASTM D 3363 – Standard Test Method for Film Hardness by Pencil Test.
8.
ASTM G 155 – Standard Practice for Operating Xenon Arc Light Apparatus for Exposure of Non-Metallic Materials.
B.
ISO Testing Standards:

1.
ISO 1520 – Paints and Varnishes – Cupping Test.
2.
ISO 2815 – Paints and Varnishes – Buchholz Indentation Test.
1.3
SUBMITTALS
A.
Product Data: Submit manufacturer’s product data, storage and handling requirements and recommendations, installation methods and available colors, styles, patterns and textures. 

B.
Shop Drawings: Submit manufacturer’s shop drawings, including plans and elevations, indicating overall dimensions.
C.
Samples:  Submit manufacturer’s samples of materials, finishes, and colors.
D.
Warranty: Manufacturer’s standard warranty.
1.4
QUALITY ASSURANCE
A.
Manufacturer’s Qualifications: Manufacturer regularly engaged in manufacture of site furnishings since 1969.
B.
Product Support: Products are supported with complete engineering drawings and design patents.

C.
Base Worth: An installed base of products worth in excess of one hundred million dollars.

D.
Assets: Excess of twenty million dollars in assets.

E.
Production: Orders are filled within a 40-day schedule.

F.
Facility Operator: Welders and machine operators are certified.
1.5
DELIVERY, STORAGE, AND HANDLING
A.
Delivery:  Deliver materials to site in manufacturer’s original, unopened containers and packaging, with labels clearly identifying product name and manufacturer.

B.
Storage: Store materials in clean, dry area in accordance with manufacturer’s instructions.  Keep materials in manufacturer’s original, unopened containers and packaging until installation.
C.
Handling:  Protect materials and finish during handling and installation to prevent damage.

1.6
WARRANTY
A.
Warranty Information:  

-Products will be free from defects in material and/or workmanship for a period of three years from the date of invoice.

-The warranty does not apply to damage resulting from accident, alteration, misuse, tampering, negligence, or abuse.

- Landscape Forms, Inc. shall, at its option, repair, replace, or refund the purchase price of any items found defective upon inspection by an authorized Landscape Forms service representative.
-Purchasers should be aware that normal use of these high quality products can result in superficial damage affecting the finish.  Scratches, nicks, and dents are to be considered normal wear and tear, and are not the responsibility of the manufacturer.
PART 2
PRODUCTS

2.1
MANUFACTURER
A.
Landscape Forms, Inc., 7800 E. Michigan Ave, Kalamazoo, Michigan 49048.                       Phone: (800) 521-2546. Fax (269) 381-3455.  Website www.landscapeforms.com                       E-mail:  specify@landscapeforms.com
2.2
SITE SEATING AND TABLES
A.
“Chipman” 
B.
Size:  
1.
31in diameter
2.
36in diameter
3.
45in diameter
4.
27in square

5.
32in square

6.
37in square

C.
Height: 

1.
Lounge table, 15in height
2.
Dining table, 29in height

3.
Standing table, 42in height (not available for 45in dia or 37in sq)
D.
Umbrella hole option: 

1.
No umbrella hole

2.
With umbrella hole (only available on dining height tables)

E.
Mounting option: 

1.
Freestanding

2.
Surface mount (dining tables with umbrella hole MUST be surface mounted)

2.3
MATERIALS
A.
Tabletop Frame: Outer table frame is type A360 or A413 cast aluminum.
B.
Tabletop insert: Tabletops are .090" thick (13 gauge) ASTM A 1011 hot rolled pickled and oiled commercial steel type B with welded on stainless steel standoffs for bolting to table frame casting. Insert is secured to tabletop frame with 1/4-20 stainless steel serrated flange locknuts.
C.
Center column: Column is type A360 or A413 cast aluminum. Column bolts to tabletop frame with (4) 5/16-18 button head cap screws, Magni-coated with washers.
D.
Table base: Table base is type A356 cast aluminum. Base bolts to center column with (1) 1/2-13 x 5” socket head cap screw, Magni-coated, with a washer.
E.     Freestanding glides: Nylon 1-1/4” diameter base, color is black, with a 5/16-18 stem, carbon steel with Magni-coat.
F.
Surface Mount Glides: All surface mount anchor components are steel with black Magni-coat. Glide base is 1-1/4” diameter, cast steel. Custom 5/16-18 screw is secured to glide base with retaining ring. Glide base threads into custom cast steel concrete anchor. (2) anchor bolts are included if specified.
2.4
RECYCLED CONTENT   All units are 100% recyclable.
	 
	Pre-Consumer Content
	Post-Consumer Content

	31in dia lounge table
	56.5%
	20.7%

	31in dia dining table
	57.4%
	19.2%

	31in dia standing table
	58.1%
	18.1%

	36in dia lounge table
	54.7%
	23.4%

	36in dia dining table
	55.6%
	22.0%

	36in dia standing table
	56.3%
	21.0%

	45in dia lounge table
	51.6%
	28.3%

	45in dia dining table
	52.5%
	27.0%

	27in square lounge table
	57.4%
	19.3%

	27in square dining table
	58.3%
	17.9%

	27in square standing table
	58.9%
	16.9%

	32in square lounge table
	56.0%
	21.5%

	32in square dining table
	56.8%
	20.3%

	32in square standing table
	57.4%
	19.4%

	37in square lounge table
	54.4%
	24.0%

	37in square dining table
	55.1%
	22.9%


2.5
FINISHES
A.
Finish on metal:  Landscape Forms, Inc. “Pangard II”.
1.
Primer:  Rust inhibitor.

2.
Topcoat:  Thermosetting polyester powdercoat.  UV, chip, and flake resistant.
3.
Test Results:  “Pangard II”.

a.
Gloss, Garner 60 Degrees, ASTM D 523:  Plus or minus 5.

b.
UV Resistance, Color and Gloss, ASTM G 155, Cycle 7:  Delta E less than 2 at 2.0 mils and less than 20 percent loss.

c.
Cross-Hatch Adhesion, ASTM D 3359, Method B:  100 percent pass.

d.
Flexibility Test, Mandrel, ASTM D 522:  3 mm at 2 mils.

e.
Erichsen Cupping, ISO 1520:  8 mm.

f.
Impression Hardness, Buchholz, ISO 2815:  95.

g.
Impact Test, ASTM D 2794:  60 inches/pound at 2.5 mils. 
h.
Pencil Hardness, ASTM D 3363:  2H minimum.

i.
Corrosion Resistance, 1,500-Hour Test, ASTM B 117:  Max undercutting 1 mm.

j.
Humidity Resistance, 1,500-Hour Test, ASTM D 2247:  Max blisters 1 mm.

Specifier Notes:  Specify color of metal finish.  Consult Landscape Forms, Inc. for selection of standard colors and availability of custom colors.
4.
Tabletop insert/frame color:  ____________
5.
Table support color:  ____________

PART 3
EXECUTION

3.1
EXAMINATION
A.
Examine areas to receive tables.
B.
Notify Architect of conditions that would adversely affect installation or subsequent use.
C.
Do not begin installation until unacceptable conditions are corrected.

3.2
INSTALLATION
A.
Install tables in accordance with manufacturer’s instructions at locations indicated on the Drawings.

B.
Install tables plumb and level.

C.
Anchor tables securely in place, if specified.

3.3
ADJUSTING
A.
Finish Damage: Repair minor damages to finish in accordance with manufacturer’s instructions and as approved by Architect.
B.
Component Damage: Remove and replace damaged components that cannot be successfully repaired as determined by Architect.

3.4
CLEANING
A.
Clean tables promptly after installation in accordance with manufacturer’s instructions.

B.
Do not use harsh cleaning materials or methods that could damage finish.

3.5
PROTECTION
A.
Protect installed tables to ensure that, except for normal weathering, tables will be without damage or deterioration at time of Substantial Completion.
END OF SECTION


Landscape Forms
Chipman Table

Page 4 of 4 
09/01/2023

